| ITEM | DESCRIPTION |  |  |  |  |  |  |  |  |  | QTY | UNIT | RATE | K.SHS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PROPOSED DEPUTY COUNTY COMMISSONER |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | RESIDENCE IN LAFEY CONSTITUENCY |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | ELEMENT NO. 1 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | SUBSTRUCTURES (ALL PROVISIONAL) |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Site clearance |  |  |  |  |  |  |  |  |  |  |  |  |
| A |  | Clear the site off bushes, grass, shrubs and all vegetation; cart |  |  |  |  |  |  |  |  | 125 | sm | 40.00 | 5,000.00 |
|  |  | away as directed. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Destroy termite nests within site works, take out and destroy |  |  |  |  |  |  |  |  |  |  |  |  |
| B |  | queens, impregnate holes and tunnels with insecticide and fill |  |  |  |  |  |  |  |  | 125 | sm | 30.00 | 3,750.00 |
|  |  | voids with approved material |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Oversite excavation |  |  |  |  |  |  |  |  |  |  |  |  |
| C |  | Excavate oversite to remove top soil average 150mm deep and |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | wheel away from site approximately 50mts |  |  |  |  |  |  |  |  | 125 | sm | 100.00 | 12,500.00 |
|  |  | Excavation and earthworks |  |  |  |  |  |  |  |  |  |  |  |  |
| D |  | Excavate to reduce levels not exceeding 1.5 m deep comencing |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | from sripped level |  |  |  |  |  |  |  |  | 125 | cm | 200.00 | 25,000.00 |
| E |  | Excavate for strip foundation trenches not exceeding 3 m deep |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | commencing from stripped level |  |  |  |  |  |  |  |  | 125 | cm | 500.00 | 62,500.00 |
| F |  | Excavate pits for column bases starting from ground level not |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | exceeding 1.50 meters. |  |  |  |  |  |  |  |  | 40 | cm | 150.00 | 6,000.00 |
| G |  | Ditto exceeding 1.5 but not exceeding 3.0metres |  |  |  |  |  |  |  |  | 30 | cm | 200.00 | 6,000.00 |
| H |  | Extra-over all excavation for excavating in rock irrespective of |  |  |  |  |  |  |  |  | 30 | cm | 600.00 | 18,000.00 |
|  |  | class |  |  |  |  |  |  |  |  |  |  |  |  |
| I |  | Return, fill-in and rum selected excavated materials around |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | foundations |  |  |  |  |  |  |  |  | 125 | cm | 150.00 | 18,750.00 |
| J |  | Remove and cart away from site surplus excavated materials. |  |  |  |  |  |  |  |  | 125 | cm | 150.00 | 18,750.00 |
|  |  | Diposal of water |  |  |  |  |  |  |  |  |  |  |  |  |
| K |  | Keeping all excavations free from all water including |  |  |  |  |  |  |  |  | 1 | Item | 30,000.00 | 30,000.00 |
|  |  | spring or running water |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Carried to Collection |  |  |  |  |  |  |  |  |  | 206,250.00 |

|  | Planking and strutting |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | Uphold the sides of all excavations |  |  | 1 |  |  | Item | 40,000.00 | 40,000.00 |
|  | Filling |  |  |  |  |  |  |  |  |
| B | 300mm thick hardcore filling in making up levels, well compacted |  |  |  |  |  |  |  |  |
|  | and rolled to layers of 150mm maximum thickness |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  | cm | 700.00 | 42,000.00 |
| C | 50mm thick murram blinding to surfaces of fill |  |  | 125 |  |  | sm | 200.00 | 25,000.00 |
|  | Antitermite treatment |  |  |  |  |  |  |  |  |
|  | Termidor' or other equal and approved anti-termite insecticide |  |  |  |  |  |  |  |  |
| D | treatment with ten years guarantee, applied strictly in accordance |  |  |  |  |  |  |  |  |
|  | with manufacturer's instructions, to tops of fill and foundation |  |  |  |  |  |  |  |  |
|  | walls |  |  | 125 |  |  | sm | 50.00 | 6,250.00 |
|  | Insitu concrete works; mass concrete 1:3:6/20/12mm |  |  |  | aggregate |  |  |  |  |
| E | 50mm thick blinding under foundations |  |  | 125 |  |  | sm | 300.00 | 37,500.00 |

Vibrated reinforced concrete (1:2:4-20-20 aggregate) as

described in

| F | Strip foundation | 7 | cm | 11,000.00 | 77,000.00 |
| --- | --- | --- | --- | --- | --- |
| G | 150mm thick ground slab | 125 | sm | 1,400.00 | 175,000.00 |
| H | Extra; splayed thicknessing to underside of bed size 300mm |  |  |  |  |
|  | average wide x 150mm thick; forming sinking in hardcore; |  |  |  |  |
|  | dressing polythene sheeting to profile and |  |  |  |  |
|  | any additional fabric reinforcement | 55 | lm | 200.00 | 11,000.00 |
| I | Ditto size 600mm wide x 150mm thick including ditto | 55 | lm | 200.00 | 11,000.00 |
|  | Carried to Collection |  |  |  | 424,750.00 |

Steel reinforcement

Supply and fix high tensile D-bars to B.S

4661reinforcement bars including bending hooks tying

| A | 16mm diameter bars | 450kg | 150.00 | 67,500.00 |
| --- | --- | --- | --- | --- |

| B | 12mm ditto |  | 300 | kg | 150.00 | 45,000.00 |
| --- | --- | --- | --- | --- | --- | --- |
| C | 8mm ditto |  | 200 | kg | 150.00 | 30,000.00 |
|  | Fabric; B.S. 4483 |  |  |  |  |  |
|  | Reference A142 mesh 200 x 200 mm , weight 2.22 kgs per square |  |  |  |  |  |

- meter ( measured net - no allowance made for laps) inclunding bends, tying wire and distance

|  | blocks |  |  |  |  |  | 125 | sm | 150.00 | 18,750.00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Sawn formwork to insitu concrete as described:- |  |  |  |  |  |  |  |  |  |
| E | To vertical sides of columns |  |  |  |  |  | 30 | sm | 800.00 | 24,000.00 |
| F | To sides of columns bases |  |  |  |  |  | 30 | sm | 800.00 | 24,000.00 |
| G | To edges of ground floor slab; 150mm girth |  |  |  |  |  | 150 | lm | 300.00 | 45,000.00 |
|  | Walling |  |  |  |  |  |  |  |  |  |
| H | 200mm thick approved local natural stone; chisel dressed square; |  |  |  |  |  |  |  |  |  |
|  | bedding, jointing and pointing in cement sand (1:4) mortar |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 125 | sm | 1,150.00 | 143,750.00 |
|  | Damp proofing |  |  |  |  |  |  |  |  |  |
|  | Polythene sheet; 500 gauge, 200mm welted laps (no allowance |  |  |  |  |  |  |  |  |  |
| J | made to laps), horizontal; 1 no. layer laid on compacted quarry |  |  |  |  |  | 155 | sm | 200.00 | 31,000.00 |
|  | dust blinding |  |  |  |  |  |  |  |  |  |
|  | 200mm wide; B.S. 743 Type A bitumen hessian base 150 mm laps |  |  |  |  |  |  |  |  |  |
| K | (no allowance made for laps); horizontal, 1 no. layer, bedded in |  |  |  |  |  | 110 | lm | 100.00 | 11,000.00 |
|  | cement sand (1:3) mortar |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Carried to Collection |  |  |  |  | 440,000.00 |
|  | Plinth areas |  |  |  |  |  |  |  |  |  |
|  | Insitu Finishings |  |  |  |  |  |  |  |  |  |
| A | 14mm thick 2 No. coatwork cement sand (1:3) render; wood |  |  |  |  |  |  |  |  |  |

|  | floated to concrete or blockwork base to walls; external |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 18 | sm | 320.00 | 6,900.00 |
|  | Painting and Decorations |  |  |  |  |  |  |  |  |
|  | Prepare and apply three coats bituminous paint to: |  |  |  |  |  |  |  |  |
| B | Wood floated rendered plinths over 300mm girth |  |  |  |  | 18 | sm | 300.00 | 4,600.00 |
|  | Paved area around the perimeter of each building block |  |  |  |  |  |  |  |  |
| C | 'Dragnet FT'' or other equal and approved chemical insecticide |  |  |  |  |  |  |  |  |
|  | treatment prepared and applied according to the manufacturer's |  |  |  |  |  |  |  |  |
|  | printed instructions. |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 36 | sm | 100.00 | 4,600.00 |
| D | 600 x 600 x 50mm Thick precast concrete paving slabs |  |  |  |  | 36 | sm | 1,200.00 | 55,200.00 |
|  |  | Carried to Collection |  |  |  |  |  |  | 71,300.00 |
|  | COLLECTION |  |  |  |  |  |  |  |  |
|  | From page BW / 1 |  |  |  |  |  |  |  | 206,250.00 |
|  | From page BW / 2 |  |  |  |  |  |  |  | 424,750.00 |
|  | From page BW / 3 |  |  |  |  |  |  |  | 440,000.00 |
|  | From above |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | 71,300.00 |

|  | TOTAL FOR SUBSTRUCTURES CARRIED TO SUMMARY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | 1,142,300.00 |
|  | ELEMENT NO. 2 |  |  |  |  |  |  |  |  |
|  | FRAME |  |  |  |  |  |  |  |  |
|  | Insitu concrete; reinforced; class 20 / (20mm); vibrated |  |  |  |  |  |  |  |  |
| A | Beams (Lintel) |  |  |  |  | 6 | cm | 11,000.00 | 66,000.00 |
| B | Columns |  |  |  |  | 4 | cm | 11,000.00 | 44,000.00 |

|  |  | Reinforcement |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Bars; high yield steel; cold worked to B.S. 4461 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | including bends, hooks, tying wire and distance blocks |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| G |  | 16mm diameter bars |  |  |  |  |  |  |  |  |  |  |  |  | 320 | kg | 150.00 | 48,000.00 |
| J |  | 8mm Ditto |  |  |  |  |  |  |  |  |  |  |  |  | 132 | kg | 160.00 | 21,120.00 |
|  |  | Sawn formwork to insitu concrete as described:- |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| K |  | To sides and soffits of beams |  |  |  |  |  |  |  |  |  |  |  |  | 180 | sm | 100.00 | 18,000.00 |
|  |  |  |  | Carried to Collection |  |  |  |  |  |  |  |  |  |  |  |  |  | 197,120.00 |
| A |  | Edge of risers; 75 to 150mm wide |  |  |  |  |  |  |  |  |  |  |  |  |  | lm |  | 43,000.00 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 172 |  | 250.00 |  |
| B |  | Edge of open strings of staircase 225 to 300mm including forming |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | profiles of treads and risers |  |  |  |  |  |  |  |  |  |  |  |  | 88 | lm | 250.00 | 22,000.00 |
|  |  |  |  | Carried to Collection |  |  |  |  |  |  |  |  |  |  |  |  |  | 65,000.00 |
|  |  | COLLECTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | From page BW / 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 197,120.00 |
|  |  | From above |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 65,000.00 |
|  |  | TOTAL FOR FRAME CARRIED TO SUMMARY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 262,120.00 |
|  |  | WALLING ELEMENT NO.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Precast concrete grade 20(20mm aggregate) including |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | formwork, finishing fair on all exposed surfaces and |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | hoisting and placing in position, bedding, jointing and |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | pointing in cement and sand (1:4) mortar |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A |  | WINDOW CILL; 250 x 50 mm extreme height; once weathered; |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | once throated; reinforced as necessary for handling; surface fair |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | finish 275 mm girth; bedding, jointing and pointing in cement |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | sand (1:3) mortar |  |  |  |  |  |  |  |  |  |  |  |  | 57 | lm | 650.00 | 37,050.00 |
|  |  | NATURAL STONE WALLING |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  | Fine chiselled natural stone walling bedded and jointed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | in cement and sand (1:3) mortar and reiforced with and |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | including 20swg x 25mm wide hoop iron at alternate courses. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| B | 200 mm thick approved local; chisel dressed both sides; bedding, |  |  |  |  |  |  |  |  |  |  |  |  |  | 300 | sm | 1,400.00 | 420,000.00 |
|  | jointing and pointing in cement sand (1:3) mortar |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Sundries |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| C | Form opening for ventilation 150mm in diameter and build in |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | ventilating gratings |  |  |  |  |  |  |  |  |  |  |  |  |  | 12 | No. | 400.00 | 4,800.00 |
|  | Labour and material |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Extra over horizontal and vertical pointing in 10mm thick rod in |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D | cement and sand (1:3) mortar; one coat of bituminous paint. |  |  |  |  |  |  |  |  |  |  |  |  |  | 684 | sm | 300.00 | 205,200.00 |
|  | TOTAL FOR WALLING |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 667,050.00 |
|  | ELEMENT NO. 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | DOORS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | EXTERNAL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Standard steel casement door |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Metal casement steel door in 50 x 45mm RHS bottom, middle and |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | top rails, 50 x 45mm RHS stiles infilled with cranked steel panels |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A | welded to frames or glazing (m/s); all framed to deatail; overall |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 | No. | 12,120.00 | 84,840.00 |
|  | size 900 x 2400mm high; single leaf |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  | INTERNAL |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Doors, frames and finishing in wrought cypress |  |  |  |  |  |  |  |
|  | Wrot Cypress framed frames and framings |  |  |  |  |  |  |  |
| B | 150 x 50 mm; 2 No. labours; plugged door frame |  |  |  | 40 | lm | 650.00 | 26,000.00 |
| C | 150 x 50 mm transome |  |  |  | 40 | lm | 650.00 | 26,000.00 |
| D | 40 x 35 mm moulded architrave |  |  |  | 40 | lm | 350.00 | 14,000.00 |
| E | 25 x 25mm moulded quadrants |  |  |  | 13 | lm | 300.00 | 3,900.00 |

45mm solid core flush door faced both sides with 3mm plywood

- veneered and hardwood lipped all round, size 900 x 2100mm. Including fanlight 600 x 600mm.

|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 | No. | 4,500.00 | 31,500.00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sundries |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Iron mongery |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Supply and fix the following to UNION catalogue or |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | other equal and approved |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | To softwood, hardwood or the like fixing with screws |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| G |  | 3 lever satin chrome lockcase 57mm centres with Phoenix |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 | No. | 4,000.00 | 28,000.00 |
|  |  | aluminium handle on plate. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| H |  | 100mm steel butt hinges |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 11 | prs | 500.00 | 5,500.00 |
|  |  |  |  |  | Carried to collection |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 219,740.00 |
|  |  | To concrete or blockwork; fixing with bolts; plugging |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A |  | Rubber door stop; oval satin anodised aluminium - floor |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 | No. | 350.00 | 2,450.00 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | - |
|  |  | Painting and Decorations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | - |
|  |  | Prepare and apply one coat etching primer, one |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | undercoat and two coats gloss oil paint to Crown Solo or |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | - |
|  |  | other equal and approved to: - |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| B |  | General surfaces of glazed casement metal doors; over 300mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | - |
|  |  | girth internal |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 16 | sm | 700.00 | 11,200.00 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | - |
|  |  | On wood |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | - |
|  |  | Aluminium primer or other equal and approved wood |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | - |
|  |  | primer before fixing: - |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| C |  | Backs of frame, board, etc over 100mm but not exceeding 200mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | lm | 150.00 | 12,000.00 |
|  |  | girth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Knot, prime and stop; prepare and apply one undercoat |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | - |
|  |  | and two coats of gloss oil paint |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | - |
| D |  | General surfaces of timber doors over 300mm girth; external |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 53 | sm | 300.00 | 15,900.00 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | - |
|  |  |  |  |  | Carried to Collection |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 41,550.00 |

|  | COLLECTION |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | From page BW / 7 |  |  |  |  |  |  |  |  |  |  |  |  | 219,740.00 |
|  | From above |  |  |  |  |  |  |  |  |  |  |  |  | 41,550.00 |
|  | TOTAL FOR DOORS CARRIED TO SUMMARY |  |  |  |  |  |  |  |  |  |  |  |  | 261,290.00 |
|  | ELEMENT NO. 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | WINDOWS |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bull-nosed burnt clay, finishing fair on all exposed |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | surfaces and hoisting and placing in position, bedding, |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | jointing and pointing in pigmented cement and sand |  |  |  |  |  |  | (1:3) mortar |  |  |  |  |  |  |
| A150 x 25mm thick clay window sill |  |  |  |  |  |  |  |  |  |  | 30 | lm | 100.00 | 3,000.00 |
| B | 6mm thick ceramic tiles finish to internal surfaces of window |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | sills; 150mm girth |  |  |  |  |  |  |  |  |  | 30 | lm | 200.00 | 6,000.00 |
|  | Wrot Mahogany |  |  |  |  |  |  |  |  |  |  |  |  |  |
| C | 150 x 25 mm thick window boards including bull-nosed edges and |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25 x 25 mm bearer; plugged, counter sinking |  |  |  |  |  |  |  |  |  | 30 | lm | 100.00 | 3,000.00 |
|  | Curtain Tracks; |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Carried to Collection | 12,000.00 |
| --- | --- |

METAL WORK - ALUMINIUM

PURPOSE - MADE UNITS

|  |  | Supply, assemble and fix the following purpose-made |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | mild steel casement windows; standard metal section |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | from approved manufacturer complete with frames, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | transomes, mullions and with and including permanent |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | ventilators comprising "T" bar, gauze and 16 gauge |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | sheet metal hood 50mm high x 50mm projection to full |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | width of window, coupling mullions, approved |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | ironmongery and one coat manufacturer's primer; all |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | welding ground to smooth finish. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Steel; for glazing with putty, lugs to two jambs, cutting and |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | pinning to concrete or blockwork, fixing to head and sill with |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | screws; plugging |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A |  | Window, overall size 2500 x 1500mm high. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 | No | 8,000.00 | 24,000.00 |
| B |  | Window, overall size 2000 x 1500mm high. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 | No | 6,800.00 | 20,400.00 |
| C |  | Window, overall size 2000 x 1900mm high. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4 | No | 4,000.00 | 16,000.00 |
| D |  | Window, overall size 1800 x 1500mm high. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 | No | 5,000.00 | 15,000.00 |
| E |  | Window, overall size 800 x 900mm high . |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 9 | No | 2,000.00 | 18,000.00 |
| F |  | Window, overall size 1000 x 1900mm high . |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4 | No | 3,150.00 | 12,600.00 |
|  |  | Grills |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | High tensile steel burglar proof bars at 300mm cross |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | centres horizontal and vertical; ends lugged in masonry. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| K |  | 12mm diameter bars welded and ground smooth to approval. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | sm | 2,000.00 | 40,000.00 |
|  |  | Glazing |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| L |  | 4mm Thick clear sheet glass panes over 0.1 but not exceeding 0.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | sm | 1,200.00 | 24,000.00 |
|  |  | square meters; fixing with putty |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| M |  | Ditto but obscured panes ditto |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 8 | sm | 2,200.00 | 17,600.00 |
|  |  |  |  |  | Carried to Collection |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 187,600.00 |
|  |  | Painting and Decorations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | On Metal work |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Prepare and apply three coats oil paint full gloss to |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Crown Solo or other equal and approved to: - |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A |  | General window surfaces; over 300mm girth. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 40 | sm | 300.00 | 12,000.00 |

| B |  | Surfaces of metal over 300mm girth (burglar proofing grills) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 40 | sm | 300.00 | 12,000.00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Carried to Collection |  |  |  |  |  |  |  |  |  |  | 24,000.00 |
|  |  | COLLECTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | From page BW / 9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 12,000.00 |
|  |  | From page BW / 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 187,600.00 |
|  |  | From above |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 24,000.00 |
|  |  |  |  |  |  |  |  |  |  |  | TOTAL FOR WINDOWS CARRIED TO |  |  |  |  |  |  |  |  | 223,600.00 |
|  |  |  |  |  |  |  |  |  |  |  |  | SUMMARY |  |  |  |  |  |  |  |  |
|  |  | ELEMENT NO. 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | FINISHES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Wall finishes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Insitu finishes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Render; 18mm thick, 1 No. coatwork of cement and |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | sand (1:3); wood floated to concrete or blockwork base |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A |  | Walls, beams and columns; external |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 46 | sm | 700.00 | 32,200.00 |
|  |  | Plaster; 18mm thick, 2 No. coatwork, 12mm first coat |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | of cement sand (1:3); 3mm second coat of cement and |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| B |  | Walls, beams and columns; internal |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 703 | sm | 300.00 | 210,900.00 |
|  |  | Ceramic Floor tiles |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| C |  | Supplly and fix ceramic tiles approved by Client and Architect |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 200 | sm | 1,550.00 | 310,000.00 |
|  |  | Skirting. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 742 | lm | 850.00 | 630,700.00 |
|  |  | 100x50mm thick wood perquette skirting |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Beds or Backings |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Render; cement and sand (1:3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | 14mm thick one coat backings; wood floated to receive ceramic |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F |  | tiles (m/s) to concrete or blockwork base; to walls internal |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 34 | sm | 450.00 | 15,300.00 |
|  |  | Painting and decorations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Prepare and apply one undercoat and three coats of first |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | quality plastic emulsion paint to: - |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| G |  | Plastered walls |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 723 | sm | 100.00 | 72,300.00 |
|  |  |  |  | Carried to Collection |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1,271,400.00 |
| A |  | Wood floated rendered surfaces |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | sm |  | 13,800.00 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 46 |  | 300.00 |  |
|  |  | Floor finishes (Wet Areas) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Tile, Slab or Block Finishings |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Imported porcelein tiles as supplied by "Tiles and Carpet |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Centre" or other equal and approved supplier; approved |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | colour to regular or other approved pattern; bedding and |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | jointing in cement sand (1:4) mortar |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| B |  | 330 x 330 x 8mm thick; butt joints both ways; to cement sand base |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | (m/s); to walls and floors; internal |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 78 | sm | 300.00 | 23,400.00 |
| C |  | 100 mm thick skirting; straight junction with wall and floor finish. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 48 | lm | 250.00 | 12,000.00 |
|  |  | Ceiling finishes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 12mm (minimum) two-coat plaster; 9mm first coat of |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | cement sand (1:6), 3mm second coat of cement and lime |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | putty (1:10); steel trowelled to: - |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D |  | Concrete soffits |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | sm | 300.00 | 15,000.00 |
|  |  | Wrot cypress |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| E |  | 75 x 25mm moulded cornice |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 103 | lm | 180.00 | 18,540.00 |
|  |  | Approved sawn softwood; second grade; pressure impregnated. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| F |  | 75 x 50mm brandering |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 700 | lm | 250.00 | 175,000.00 |
| G |  | 75 x 50mm ditto; plugged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | lm | 350.00 | 24,500.00 |
|  |  | GYSPSUM FINISHINGS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| H |  | 18mm thick ceiling board fixed to aluminum hanges (m/s); |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | punched and puttied |  |  |  |  |  |  |  |  |  |  | 139 | sm | 350.00 | 48,650.00 |
|  |  |  |  |  | Carried to Collection |  |  |  |  |  |  |  |  |  |  | 330,890.00 |
|  |  | Extra; access panels 600 x 600mm, 50 x 50mm softwood framing |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A |  | all round; fixing with chromium plated |  |  |  |  |  | screws and cup washers |  |  |  |  | 3 | No. | 1,600.00 | 4,800.00 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | - |
|  |  | Painting and Decorations |  |  |  |  |  |  |  |  |  |  |  |  |  | - |
|  |  | Prepare and apply one undercoat and three coats of first |  |  |  |  |  |  |  |  |  |  |  |  |  | - |
|  |  | quality emulsion paint to the following surfaces |  |  |  |  |  |  |  |  |  |  |  |  |  | - |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | - |
| B |  | Steel trowelled surfaces |  |  |  |  |  |  |  |  |  |  | 165 | sm | 200.00 | 33,000.00 |
| C |  | ceiling boards |  |  |  |  |  |  |  |  |  |  | 139 | sm | 200.00 | 27,800.00 |
| D |  | Moulded cornice not exceeding 100mm girth |  |  |  |  |  |  |  |  |  |  | 207 | lm | 90.00 | 18,630.00 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | - |
|  |  |  |  |  | Carried to Collection |  |  |  |  |  |  |  |  |  |  | 84,230.00 |
|  |  | COLLECTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | From page BW / 12 |  |  |  |  |  |  |  |  |  |  |  |  |  | 1,271,400.00 |
|  |  | From page BW / 13 |  |  |  |  |  |  |  |  |  |  |  |  |  | 330,890.00 |
|  |  | From page above |  |  |  |  |  |  |  |  |  |  |  |  |  | 84,230.00 |
|  |  | TOTAL FOR FINISHES CARRIED TO SUMMARY |  |  |  |  |  |  |  |  |  |  |  |  |  | 1,686,520.00 |
|  |  | ELEMENT NO.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | ROOFING CONSTRUCTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Corrugated steel roof covering at 30 degrees pitch |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Roof covering; 150mm laps on one end and one and a half |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A |  | corrugation side lap; and nailing to 75 x 50mm celcured sawn |  |  |  |  |  |  |  |  |  |  | 150 | sm | 750.00 | 112,500.00 |
|  |  | cypress purlins (m/s) to receive interlocking tiles (m/s) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| B |  | Extra over for raking cutting |  |  |  |  |  |  |  |  |  |  | 46 | lm | 350.00 | 16,100.00 |
|  |  | Accessories, fixing as necessary to roof sheets |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CARPENTRY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The following in sawn celcured cypress timber trusses; nailed connections, spanning, hoisting 14.40m above ground

- 150 x 50mm rafters and tie beam

- 100 x 50mm struts and ties

- 150 x 50mm king-post

F	150 x 25mm nailing plates

The following in sawn celcured cypress

- 100 x 50mm wall plate rag-bolted at 1200mm centres with 12mm diameter bolts (m/s)

- 200 x 40mm ridge board

| I | 75 x 50mm purlins |
| --- | --- |

Carried to Collection.

The following in wrot cypress

- 200 x 25mm thick facsia/barge board

25 x 100mm Tongue & Grooved boarding secret nailed to and

- including 50 x 50mm celcured wrot cypress bearers or brandering at 600mm centres

Labour and material (Provisional)

12mm diameter rag bolt, 200mm long complete with and

- including head, nut and two washers; cast into and including mortice in stone or concrete and run with cement and sand (1:3) mortar

- Drill 50mm thick timber to receive 12mm dia. Bolt

RAINWATER DISPOSAL

26g Galvanised mild steel sheeting

Raking flashing overall 400mm girth with top end turned and wedged into and including 50 x 50mm mortice in stone or

- concrete and rin in with cement and sand (1:3) mortar and bottom edge dressed over roofing

| 300 | lm | 90.00 | 27,000.00 |
| --- | --- | --- | --- |
| 150 | lm | 70.00 | 10,500.00 |
| 91 | lm | 300.00 | 27,300.00 |
| 60 | lm | 320.00 | 19,200.00 |

| 84 | lm | 200.00 | 16,800.00 |
| --- | --- | --- | --- |
| 49 | lm | 250.00 | 12,250.00 |
| 134 | lm | 150.00 | 20,100.00 |
|  |  |  | 261,750.00 |

| 49 | lm | 400.00 | 19,600.00 |
| --- | --- | --- | --- |
| 92 | sm | 1,100.00 | 101,200.00 |

| 62 | No | 100.00 | 6,200.00 |
| --- | --- | --- | --- |
| 62 | No. | 50.00 | 3,100.00 |

| 50 | lm | 350.00 | 17,500.00 |
| --- | --- | --- | --- |

|  |  | 24g Galvanised mild steel sheeting; all welds ground smooth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 150 x 100mm box gutter including soldered joints in the running |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| E |  | length fixed to fascia board with and including brackets at |  |  |  |  |  |  |  |  |  |  |  |  | 74 | lm | 450.00 | 33,300.00 |
|  |  | approved centers |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| F |  | Extra; ends |  |  |  |  |  |  |  |  |  |  |  |  | 12 | No | 350.00 | 4,200.00 |
| G |  | Extra; 100mm dia.outlet 100mm long |  |  |  |  |  |  |  |  |  |  |  |  | 12 | No | 300.00 | 3,600.00 |
| H |  | 100mm dia. rainwater downpipe fixed with and including mild |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | steel straps at 900mm centres, plugged and screwed to wall |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 178 | lm | 400.00 | 71,200.00 |
| I |  | Extra; swanneck bend with 1135mm projection |  |  |  |  |  |  |  |  |  |  |  |  | 6 | No | 300.00 | 1,800.00 |
| J |  | Extra; horse shoe bend |  |  |  |  |  |  |  |  |  |  |  |  | 6 | No | 300.00 | 1,800.00 |
|  |  |  |  |  | Carried to Collection. |  |  |  |  |  |  |  |  |  |  |  |  | 263,500.00 |
|  |  | Painting and Decorations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | On Woodwork |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Prepare and apply one zinc plumbate primer and three |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | coats of 'CROWN SOLO' or other equal and approved |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | super gloss oil paint to:- |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A |  | Fascias; 200 to 300mm girth; external |  |  |  |  |  |  |  |  |  |  |  |  | 48 | lm | 150.00 | 7,200.00 |
|  |  | On Metal work |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| B |  | General surfaces of gutters; over 300mm girth external |  |  |  |  |  |  |  |  |  |  |  |  | 76 | sm | 300.00 | 22,800.00 |
| C |  | Small pipes |  |  |  |  |  |  |  |  |  |  |  |  | 36 | lm | 150.00 | 5,400.00 |
|  |  | Tongue & Grooved timber surfaces |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Prepare and apply three coats of 'CROWN SOLO' or |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | other equal and approved polyurethane varnish to:- |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D |  | Surfaces of eaves; over 300mm girth external |  |  |  |  |  |  |  |  |  |  |  |  | 92 | sm | 300.00 | 27,600.00 |

Carried to collection

COLLECTION

From page BW / 14

From page BW / 15

From above

TOTAL FOR ROOF CONSTRUCTION.

63,000.00

261,750.00

263,500.00

63,000.00

588,250.00

| ITEM |  | DESCRIPTION |  | QTY | UNIT | RATE | K.SHS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | RESIDENCE IN LAFEY CONSTITUENCY |  |  |  |  |  |  |
|  |  | SUMMARY |  |  |  |  |  |
| 1 |  | SUBSTRUCTURES (PROVISIONAL) |  |  |  |  | 1,142,300.00 |
| 2 |  | FRAME |  |  |  |  | 262,120.00 |
| 3 |  | WALLING |  |  |  |  | 667,050.00 |
| 5 |  | DOORS |  |  |  |  | 261,290.00 |
| 6 |  | WINDOWS |  |  |  |  | 223,600.00 |
| 7 |  | FINISHES |  |  |  |  | 1,686,520.00 |
| 8 |  | ROOF CONSTRUCTION |  |  |  |  | 588,250.00 |
| 9 |  | ALLOW FOR ELECTRICAL WORKS |  |  |  |  | 200,000.00 |
| 10 |  | ALLOW FOR PLUMBING & DRAINAGE WORKS |  |  |  |  | 100,000.00 |
| 11 |  | ALLOW FOR CONTINGENCIES |  |  |  |  | 50,000.00 |
|  |  |  | TOTAL EXPENDITURE |  |  |  | 5,181,130.00 |