- KM-2560

## GENERAL

| General type | Monochrome copier for A3 |
| --- | --- |
| Technology | KYOCERA Laser, Mono Component |
| Engine speed | Up to 25/12 pages per minute A4/A3 |
| Resolution | 600 x 600 dpi (copy), 1,800 dpi level x 600 dpi (copy, photo mode), 1,200 dpi quality (print) |
| Warm-up time | 29 seconds or less from power on |
| Time to first print | Approx. 4.7 seconds |
| Time to first copy | Approx. 4.7 seconds |
| Dimensions (W x D x H) | Main unit: 570 x 620 x 762,2 mm |
| Weight | Main unit incl. toner: Approx. 75 kg |
| Power consumption | Printing: 560 W, Stand-by: 160 W, Sleep-mode: 8.7 W |
| Power source | AC 220 V ~ 240 V, 50/60 Hz |
| Noise (ISO 7779) | Printing/copying: 64.7 dB(A), Stand-by: 47 dB(A), Power-Saving (ECOpower): Immeasurably low |
| Safety standards | TÜV/GS, CE - This product is manufactured according to ISO 9001 quality standard and ISO 14001 environmental standard. |

## PAPER HANDLING

| Input capacity | 2 x 500 sheet universal paper feeder,60-120 g/m², A3-A5R, Folio 200-sheet multi-purpose tray, 45-200 g/m², A3-A6R, Folio, envelopes |
| --- | --- |
| Max. input capacity with options | 2,200 sheets A4 |
| Duplex unit | Double-side printing as standard; A3-A5, 60-80 g/m² |
| Max. output capacity | 250 sheets face-down |
| General paper handling | All paper capacities quoted are based on a paper thickness of max. 0.11 mm. Please use paper recommended by Kyocera under normal environmental conditions. |

## PRINT

| Processor | PowerPC 750CXr / 400MHz |
| --- | --- |
| Memory | Standard 512 MB (max. 1 GB) + 80 GB HDD (system shared) |
| Controller language | PRESCRIBE IIe |
| Emulations | PCL 6 (5e/XL), KPDL3 (Postscript 3 compatible), Line Printer, IBM Proprinter X24, EPSON LQ 850, DIABLO 630, KC GL (HP-7550A) |
| Fonts | 80 outline fonts (PCL6), 136 PostScript KPDL 3 fonts, 1 bitmap font, 45 types of one-dimensional barcodes plus two-dimensional barcode (PDF-417) |
| Downloadable fonts | Kyocera, PCL + TrueType format, Type 1 + 3 format |
| Features | EMPS, Proof and hold, Private print, Job storage, Job management functionality |
| Applicable OS | All current Windows operating systems, Mac OS X Version 10.2 or higher, UNIX, LINUX as well as other operating systems on request |

## COPY

| Max. original size | A3 |
| --- | --- |
| Digital copy features | Scan once print many, Electronic sort, Rotate sort, Rotate copy, n-up copy, Split copy, Cover mode, Page numbering, Margin shift, Border erase, Box image overlay, Interrupt ouput job |
| Exposure modes | Auto, manual: 7 or 13 steps |
| Preset magnification ratios | 5 Reductions/ 5 Enlargements |
| Zoom range | 25 - 400 in 1% steps (with DP-670: 25-200%) |
| Continuous copy | 1 - 999 |
| Memory capacity | 512 MB (max. 1 GB) + 80 GB HDD |
| Image adjustments | Text, Text + Photo, Photo |

## SCAN

| Bundled software | KM-NET Viewer,TWAIN source |
| --- | --- |
| Compression format | B/W: G4 (MMR), Colour: JPG |
| File type | Mono: TIFF, PDF (high compressive/encrypted), Colour: JPG, PDF (high compressive/encrypted), TIFF |
| Original recognition | Text, Photo, Text + Photo, OCR |
| Max. scan size | A3 |
| Scan functionalities | Scan-to-SMB, Scan-to-email, Scan-to-BOX, BOX TWAIN, Network TWAIN |
| Scan resolution | 600, 400, 300, 200, 200x100, 200x400, 256 greyscales, 256 colour: Colour scanning at resolutions of 400 dpi and above requires additional memory. |
| Scan speed | 50 originals per minute in b/w (300 dpi/A4), 15 originals per minute in colour (300 dpi/A4) |

## FAX

| Compatibility | ITU-T Super G3 |
| --- | --- |
| Modem speed | 33.6 kbps |
| Transmission speed | 3 or less |
| Scanning density | Fine: 8 dot/mm x 7.7 line/mm, Normal: 8 dot/mm x 3.85 line/mm, Superfine: 8 dot/mm x 15.4 line/mm, Ultrafine: 16 dot/mm x 15.4 line/mm, SUFine: 600 x 600 dpi |
| Max. original size | A3 |
| Compression method | MH, MMR, MR, JBIG |
| Memory | Standard 120 MB (enable fax image back up BU-10) |
| Fax features | Internet fax, Network faxing, Rotate transmission, Confidential, Relay and bulletin board transmission and reception |

## INTERFACES

| Standard interface | USB 2.0 (Hi-Speed), USB Host Interface dedicated for USB Flash Memory device, Fast Ethernet 10-Base-T/ 100Base-TX, CF slot |
| --- | --- |