REPUBLIC OF KENYA

MINISTRY OF ROADS, TRASPORT & PUBLIC WORKS

PUBLIC WORKS DEPARTMENT

PROPOSED

CHAINLINK FENCE & GATE HOUSE

(Concrete Posts)

AT

RHAMU DIMTU BOYS SECONDARY SCHOOL

BANISA CONSTITUENCY

BILLS OF QUANTITIES (ESTIMATE COST)

Prepared: -                                                                     Issued: -

Sub-County Works Officer,					  Fund Manager, NG-CDF

Banisa Sub-County,				                         Banisa Constituency

Department of Public Works 		                       Development Fund,

P.O. Box 49-70300,				                        P.O. Box 37-70303,

MANDERA                                                                      TAKABA

SEPTEMBER, 2020

BILL NO. 01: BILLS OF QUANTITIES FOR THE PROPOSED CHAIN-LINK FENCE

AT RHAMU DIMTU BOYS SECONDARY SCHOOL (A=960 X 780M)

| ITEM | DESCRIPTION | QTY | UNIT | RATE | SHS. |
| --- | --- | --- | --- | --- | --- |
| A B C D E F | Bush clearing Clear fence route of all bushes, shrubs, undergrowth, trees and burn or remove from site all arising including heavy bush clearing inside the compound. Concrete Posts Fencing Provide materials and erect chain link fence 2.40m high 14 gauge chain link complete with 12 ½ gauge x 6 strand galvanized barbed wire consisting of 125x 100mm cranked reinforced concrete (1:2:4 -20mm) posts at 3.0m centres and reinforced with 4No.6mm mild steel rods and 6mm mild steel stirrups at 250mm centers’ with bottom ends of post embedded in and including plain concrete 1:3:6 (40mm) base size 300mm x 300mm x 600mm deep including all excavations. Extra over for straining posts at 500m spacing supported by 2No. precast reinforced concrete (1:2:4 -20mm) raking strut size 100mm x 100mm x 1800mm long with top end bolted to fence with and including 12mm diameter galvanized steel bolt Extra over for double corner posts each with raking strut as before described and 3No. 150mm x 6mm galvanized hook bolts for straining wires Excavate trench 150mmx150mm deep along the fence and dispose on site as directed 150mmx150mm mass concrete (1:4:8) blinding to hold chain link at bottom or any other approved. | 7000 3468 60 4 78 78 | SM LM NO NO CM CM | 15 1950 2000 4000 300 5000 | 105,000 6,762,600 120,000 16,000 23,400 390,000 |
|  | TOTAL CARRIED TO COLLECTION |  |  |  | 7,417,000 |

CF/01

| ITEM | DESCRIPTION |  | QTY | UNIT | RATE | SHS. |
| --- | --- | --- | --- | --- | --- | --- |
| A B C D E | Extra over for single corner posts each with raking strut as before described and 3No. 150mm x 6mm galvanized hook bolts for straining wires GATE Provide all materials and fabricate double gate overall size 4.0m x 2.0m high comprising of 40x 25 mm x 3mm hollow section main frame and middle rail with 25x 25mm x 3mm hollow section vertical rails welded at approved design, 14SWG sheet metal welded both sides including one coat red oxide under coat and two coats external quality crown paint and mounted appropriately on to 2No. RHS 100mm square tube x 3000mm overall height galvanized mild steel pipe column with cross bars welded at bottom and embedded in 800m x 800mmx450mm thick concrete (1:2:4) (20mm) bases reinforced with 12mm square twisted reinforcement bars both ways at 200mm centres including all excavations and painting in crown external quality oil to gates and steel posts. Ditto but pedestrian gate size 1500mm x 2000mm ditto. EXCAVATION & EARTHWORKS Hardcore Filling Approved hardcore filling including leveling, consolidating and or hand packing in layers 150mm thick in storm water trenches. HEDGING Carefully return along the fence selected shrubs as directed by client user. |  | 2 1 1 NIL 3468 | NO NO NO CM LM | 2000 150000 45000 00 15 | 4,000 150,000 45,000 00 52,020 |
|  |  | TOTAL CARRIED TO COLLECTION |  |  |  | 251,020 |

CF/02

| ITEM | DESCRIPTION | QTY | UNIT | RATE | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| A B C D E F G H | GATE SIDE WALLS Excavate trench for strip foundation Return, fill and ram Spread on site surplus excavated materials Concrete(1:2:4) in foundation 200mm natural stone walling 300mm x 100mm concrete coping Cement/sand rendering to surfaces of walling Three (3) coats of plastic emulsion paint on rendered walls | 6 3 3 1 27 9 45 45 | CM CM CM CM SM LM SM SM | 300 300 300 14000 1500 250 400 350 | 1,800 900 900 14,000 40,500 2,250 18,000 15,750 |
|  | TOTAL CARRIED TO COLLECTION COLLECTION Total from page CF/01……………………… Total from page CF/02 ……………… Total from page CF/03 above……………… |  |  |  | 94,100 |
|  |  |  |  |  | 7,417,000 251,020 94,100 |
|  | TOTAL FOR FENCING AND GATES CARRIED TO GRAND SUMMARY |  |  |  | 7,762,120 |

CF/03

BILL NO. 02:  BILLS OF QUANTITIES FOR THE PROPOSED GATE HOUSE

| ITEM | DESCRIPTION | QTY. | UNIT | RATE | SHS. |
| --- | --- | --- | --- | --- | --- |
| A B C D E F G H I J K L | GUARD HOUSE(PITCHED ROOF) SUBSTRUCTURES Allow for upholding the sides of excavations. Allow for keeping the excavations out of all kind of water by pumping or otherwise. Excavate to reduce level. Excavate top vegetable soil 200mm deep and deposit onsite not exceeding 100m away. Excavate normal soil from stripped level for trench not exceeding 1.5m deep. Extra over excavation for excavating in rock. Return fill and ram selected excavated material. Cart away extra excavated material as will be directed on site. Vibrated reinforced concrete 1:2:4 to foundations. 50mm (1:4:8) concrete blinding under foundations. 200mm thick masonry walling in cement/sand mortar (1:3) Approved insecticide treatment applied with manufacturer’s instructions ‘Gradiator or other equally approved. | - - 4 8 8 1 5 6 1 6 14 12 | ITEM ITEM CM SM CM CM CM CM CM SM SM SM | 1000 1000 150 150 300 1500 300 300 14000 600 1500 350 | 1,000 1,000 600 1,200 2,400 1,500 1,500 1,800 14,000 3,600 6,000 4,200 |
|  | - TOTAL CARRIED TO COLLECTION |  |  |  | 38,800 |

GH/1

| ITEM | DESCRIPTION | QTY. | UNIT | RATE | SHS. |
| --- | --- | --- | --- | --- | --- |
| A B C D E F G H I J K | 12mm diameter square twisted bars. 8mm ditto 300mm thick approved hardcore well watered and compacted in 100mm layers. 50mm thick murram blinding on top of compacted hardcore. 500 gauge damp proof membrane including end laps Damp proof course 200mm wide including end laps. 100mm thick vibrated reinforced concrete in slab. BRC reinforcement mesh ref.No.A98 sawn formwork to edge of floor slab 75 -150 high. 12mm thick cement/sand (1:4) rendering to building. 50mm in-situ concrete paving all around the building. Prepare and apply 3 coats of black bituminous paint to plinth. | 39 15 4 12 12 12 9 12 12 13 6 | KGS KGS CM SM SM LM SM SM SM SM SM | 200 200 1500 150 150 150 12000 350 400 800 350 | 7,800 3,000 6,000 1,800 1,800 1,800 108,000 4,200 4,800 10,400 2,100 |
|  | Total carried to collection COLLECTION Total from GH/1 Total brought down from above |  |  |  | 151,700 |
|  |  |  |  |  | 38,800 151,700 |
|  | - TOTAL FOR (SUBSTRUCTURES) CARRIEDTO SUMMARY |  |  |  | 190,500 |

GH/2

| ITEM | DESCRIPTION | QTY. | UNIT | RATE | SHS. |
| --- | --- | --- | --- | --- | --- |
| A B C D E F G H I J K | WALLING AND FRAME WORK 150mm thick walling in cement/sand 1:4 mortar Reinforced concrete 1:2:4 in ring beam 12mm diameter square twisted high tensile bars. 8mm ditto Sawn formwork to ring beam Extra over ditto for fair face Precast in permanent vents 300x225mm high with mosquito gauze Extra over walling for smooth stone dressing. Ditto for smooth vertical joints and recessed horizontal joints. 12mm thick cement and sand render with wood floor finish. Construct concrete work top 500x 2500mm with reinforced concrete all reinforcement’s bars and form work inclusive. | 36 1 56 27 13 12 4 48 48 7 2 | SM CM KGS KGS SM SM SM SM SM SM SM | 1200 14000 200 200 900 450 300 350 300 400 10000 | 43,200 14,000 11,200 5,400 11,700 5,400 1,200 16,800 14,400 2,800 20,000 |
|  | TOTAL FOR WALLING CARRIED TO SUMMARY |  |  |  | 146,100 |

GH/3

| ITEM | DESCRIPTION | QTY | UNIT | RATE | SHS. |
| --- | --- | --- | --- | --- | --- |
| A B C D E F G | ROOFING Construction All timber to be sawn cypress of G.S grade seasoned to equilibrium moisture content between 9% and 15% and to a requirement of K.S 02771 of 1991 treated with approved wood preservative. 100x50mm wall plate on and including 10mm cement and sand (1:4) mortar bed secured to reinforced concrete ring beam (m.s) by mild steel anchor bolts(m.s) The following in 56No. Nailed triangular timber trusses spanning clear 12700mm height 2500mm including hoisting and placing 3500mm above floor slab level 100x50mm trussed rafter 100x50mm tie beam 100x50mm strut/tie 75X50mm sawn celcured cypress purlins Ditto hip rafters Covering 30G roofing sheets Gauge 30 coloured corrugated iron sheets (MRM works) | 18 16 9 22 43 16 28 | LM LM LM LM LM LM SM | 350 350 350 350 250 250 1050 | 6,300 5,600 3,150 7,700 10,750 4,000 29,400 |
|  | TOTAL CARRIED TO COLLECTION |  |  |  | 66,900 |

GH/4

| ITEM | DESCRIPTION | QTY. | UNIT | RATE | SHS. |
| --- | --- | --- | --- | --- | --- |
| A B C D E | Ditto hip covers 450mm girth. Ditto ridge covers 300mm girth. Wrot cypress 25x200mm fascia board fixed to ends of rafters (m.s). Sundries 12mm diameter x 300mm long black mild steel anchor bolt embedded 180mm deep in ring beam at 1200mm including drilling holes in timber. 100x100x6mm thick galvanized mild steel nailing cleat once bent to form angle ten times drilled, one flange nailed to foot of rafter(m.s) other nailed to top of wall plate(m.s) | 8 5 22 14 10 | LM LM LM NO NO | 450 300 500 250 250 | 3,600 1,500 11,000 3,500 2,500 |
|  | Carried to collection COLLECTION Total from GH/4 ………………………. Total from above ……………………… |  |  |  | 22,100 |
|  |  |  |  |  | 66,900 22,100 |
|  | TOTAL FOR ROOFING CARRIED TO SUMMARY |  |  |  | 89,000 |

GH/5

| ITEM | DESCRIPTION | QTY | UNIT | RATE | SHS |
| --- | --- | --- | --- | --- | --- |
| A B C D E F G | ELEMENT NO. 4 DOORS AND WINDOWS DOORS 1000x2100mm single leaf, outward-opening steel fabricated Paneled door complete with all hanging and fastening iron monger. WINDOWS Supply and fix Grilles window size 1500mm x 1200mm made of 20mmx20mmx2.0mm thick (1.13kg/m) Square Hollow Section tubes welded at 300mm centres both ways including ironmongeries. Ditto but weld mesh wire welded on to grilles windows framework. PAINTING Prepare and apply three coats high gloss paint on steel surfaces internally & externally Knot, prime and stop and apply three coats high gloss on fascia board. Ditto not exceeding 100-200mm girth. Doors both sides | 1 2 4 4 30 9 4 | NO NO SM SM LM LM SM | 10000 5000 600 400 150 100 400 | 10,000 10,000 2,400 1,600 4,500 900 1,600 |
|  | TOTAL OF ELEMENT NO.4 DOORS AND WINDOWS CARRIED TO SUMMARY |  |  |  | 31,000 |

GH/6

| ITEM | DESCRIPTION | QTY. | UNIT | RATE | SHS. |
| --- | --- | --- | --- | --- | --- |
| A B C D | FINISHES 12mm thick cement and sand plaster to walling. Prepare and apply 3 coats of plastic emulsion paint to walling. 40mm thick coloured screed finish steel trowelled smooth in red oxide. Prepare and apply 3 coats of oil paint to windows including burglar proofing | 72 37 11 6 | SM SM SM SM | 400 350 450 400 | 28,800 12,950 4,950 2,400 |
|  | TOTAL CARRIED TO SUMMARY SUMMARY Total from GH/2 (Substructures) Total from GH/3 (Walling/frame) Total from GH/5 (Roofing) Total from GH/6 (Doors/windows) Total from GH/7 above (Finishes) |  |  |  | 49,100 |
|  |  |  |  |  | 190,500 146,100 89,000 31,000 49,100 |
|  | TOTAL FOR GUARD HOUSE CARRIED TO GRAND SUMMARY |  |  |  | 505,700 |

GH/7

BILL NO. 03: PRIME COSTS AND PROVISIONAL SUMS

| ITEM | DESCRIPTION | SHS |
| --- | --- | --- |
| A B | PRIME COST AND PROVISIONAL SUMS PROVISIONAL SUMS Allow a Provisional Sum of Kenya Shillings Two Hundred Thousand (Kshs.200,000.00) for contingencies. Allow A Provisional Sum of Kenya Shillings Fifty Thousand (Kshs.50,000.00) only for Project Management and Documentation. | 200,000 50,000 |
|  | TOTAL FOR PC & PROVISIONAL SUMS CARRIED TO GRAND SUMMARY | 250,000 |

PC&PS/01

BILLS OF QUANTITIES FOR THE PROPOSED CONSTRUCTION OF CHAINLINK FENCE & GATE HOUSE AT RHAMU DIMTU BOYS SECONDARY SCHOOL

IN BANISA CONSTITUENCY.

GRAND SUMMARY

| ITEM | DESCRIPTION | For Official use only |  | For Contractor use only |  |
| --- | --- | --- | --- | --- | --- |
| A B. C | GRAND SUMMARY Chain link Fence Gate House PC and provisional sums | 7,762,120 505,700 250,000 | 00 00 00 |  |  |
|  | TOTAL ESTIMATED COST | 8,517,820 SAY: 8,518,000 | 00 00 |  |  |

AMOUNT IN WORDS

Kenya Shillings Eight Million, Five Hundred Eighteen Thousand

(Kshs. 8,518,000.00) only.

GS/01