REPUBLIC OF KENYA

MINISTRY OF ROADS, TRANSPORT & PUBLIC WORKS

DEPARTMENT OF PUBLIC WORKS

PROPOSED

CONSTRUCTION OF PERIMETER WALL & SECURITY HOUSE

AT

DEPUTY COUNTY COMMISSIONER’S RESIDENCE

MANDERA NORTH CONSTITUENCY

# BILLS OF QUANTITIES ESTIMATE COST

Prepared: -                                                   	               Issued: -

Sub County Works Officer,				  	Fund Account Manager,

Mandera North Sub County,					NG- Constituency Development Fund

P.O. Box 49-70300,				                        P.O. Box 23-70302

MANDERA                                                     	            RHAMU

OCTOBER, 2020

BILL NO. 01: BILLS OF QUANTITIES FOR THE PROPOSED CONSTRUCTION OF PERIMETER WALL & SECURITY HOUSE AT DEPUTY COUNTY COMMISSIONER’S RESIDENCE

| ITEM | DESCRIPTION | QTY. | UNIT | RATE | SHS. |
| --- | --- | --- | --- | --- | --- |
| A B C D E F G H | (BILL NO.01: PERIMETER WALL) SIZE 80 X 44M=248LM SUBSTRUCTURES (PROVISIONAL) SITE CLEARANCE: Clear the site of all bushes, shrubs, undergrowth and small trees grab uproot, cart away and burn all arising (2M) Prune or uproot selected “mathenge” trees or any other on site, burn or cart away the arising as directed. Excavate for strip foundation trench not exceeding 1.5m deep starting from reduced level and stockpile on site as directed. Excavate pit for pier bases. Extra over excavations for excavating in rock. Allow for maintaining and supporting sides of excavations. Allow for keeping excavations free of water and mud by pumping, bailing or other approved means Chemical anti-termite treatment (approved) on foundations. | 492 - 136 21 55 - - 148 | SM ITEM CM CM CM ITEM ITEM SM | 30 10000 300 300 1500 10000 10000 350 | 14,760 10,000 40,800 6,300 82,500 10,000 10,000 51,800 |
|  | - TOTAL CARRIED TO COLLECTION |  |  |  | 226,160 |

PW/01

| ITEM | DESCRIPTION | QTY. | UNIT | RATE | SHS. |
| --- | --- | --- | --- | --- | --- |
| A B C D E F G H I J | Return, fill and ram selected excavated material around foundations. Load, wheel and deposit surplus material where directed not exceeding 50m away Concrete work Mass concrete 1:3:6 as described in:- 50mm blinding Vibrated, reinforced concrete 1:2:4 as described in: Strip foundations Column/pier bases Foundation columns. Reinforcement mild steel square twisted reinforcement in structural concrete work 8mm diameter bars (Foundations) 10mm diameter bars (Foundations) 12mm diameter bars (beams & columns) Sawn formwork to:- Sides of vertical columns and column/pier bases. | 127 85 148 23 NIL NIL 225 516 NIL NIL | CM CM SM CM CM CM KGs KGs KGs SM | 300 300 600 14000 14000 14000 200 200 200 900 | 38,100 25,500 88,800 322,000 00 00 45,000 103,200 00 00 |
|  | - TOTAL CARRIED TO COLLECTION |  |  |  | 622,400 |

PW/02

| ITEM | DESCRIPTION | QTY. | UNIT | RATE | SHS. |
| --- | --- | --- | --- | --- | --- |
| A B C D E F | Walling in natural coursed stone from an approved quarry site jointed in 1:3 mortar 225mm thick walling in foundations Extra over walling for stone piers size 450 x 225x 1050mm high as before described. Allow for diameter 150mm weeps holes as directed. 25mm cement/sand 1:3 plinth render on concrete or masonry walls both sides. Prepare and apply 2 coats of bituminous paint to rendered surfaces. Allow concrete blocks toothing for security house walling and floor slab. | 243 NIL 30 288 288 - | SM SM NO SM SM ITEM | 1500 00 300 400 350 5000 | 364,500 00 9,000 115,200 100,800 5,000 |
|  | Total carried to collection ELEMENT NO.1 SUBSTRUCTURES COLLECTION Total brought forward from page PW/01 Total brought forward from page PW/02 Total brought forward from above page PW/03 |  |  |  | 594,500 |
|  |  |  |  |  | 226,160 622,400 594,500 |
|  | TOTAL FOR SUBSTRUCTURES CARRIED TO SUMMARY |  |  |  | 1,443,060 |

PW03

| ITEM | DESCRIPTION | QTY. | UNIT | RATE | SHS. |
| --- | --- | --- | --- | --- | --- |
| A B C D E F G H I J | 2.R. C. FRAME & WALLING Vibrated reinforced concrete (1:2:4) in columns (225 x 450mm). Ditto in beams (200 x 250mm) Square twisted M.S reinforcement in structural concrete work:- 8mm diameter bars 10mm diameter bars Sawn formwork to sides of columns and Beams 12 mm bars Walling in quarry natural stones:- 150mm thick walling (2400mm high) Raking and cutting 225mm thick stone walling for slope. 75mm thick In-situ concrete coping on top of walling, stone piers columns as directed. Razor wire Supply and fix on concrete coping resin-cot coloured coiled 450mm diameter stainless razor wire on top of walling including gate. | NIL 13 225 679 146 NIL 573 242 73 248 | CM CM KGs KGs SM KGs SM LM SM LM | 14000 14000 200 200 900 200 1200 300 1200 600 | 00 182,000 45,000 135,800 131,400 00 687,600 72,600 87,600 148,800 |
|  | TOTAL FOR R. C. FRAME & WALLING CARRIED TO SUMMARY |  |  |  | 1,490,800 |

PW/04

| ITEM | DESCRIPTION | QTY. | UNIT | RATE | SHS. |
| --- | --- | --- | --- | --- | --- |
| A B C D E F G | 3. FINISHES Extra over for neat recessed horizontal joints keying in cement/sand (1:3) mortar both sides. Cement/lime grout Prepare and apply cement/lime mortar on walling surfaces as directed. 25mm thick cement/sand render with wood float finish on concrete beams and columns and coping beams Painting Prepare and apply one undercoat and two finishing coats exterior quality silk vinyl paint on rendered concrete surfaces and coping stones. Ditto but 1st quality gloss bituminous oil paint on recessed joints GATES Provide all materials and fabricate decorative double gate overall size 4500 x 2100mm high comprising of 40x 25 mm x 3mm hollow section main frame and middle rail with 25x 25mm x 3mm hollow section vertical rails welded, 14SWG sheet metal welded both sides including one coat red oxide under coat and two coats external quality crown paint and mounted appropriately on to 2No. 300mm x 300mm x 3000mm overall height concrete 1:2:4 (20mm) reinforced with 4No. 12mm square twisted mild steel bars and 6mm stirrups at 200mm centres including 1200m x 1200mmx250mm thick concrete (1:2:4) (20mm) bases reinforced with 12mm square twisted reinforcement bars both ways at 200mm centres including all excavations formwork and rendering to all exposed surfaces and painting in crown external quality paint including formwork Ditto pedestrian gate 900mm x 2100mm high mounted on R.C columns to the approval of the Project Manager. | 1008 1008 381 381 3136 1 1 | SM SM SM SM LM NO NO | 200 300 400 350 25 150000 25000 | 201,600 302,400 152,400 133,350 78,400 150,000 25,000 |
|  | TOTAL FOR FINISHES/GATES CARRIED TO SUMMARY |  |  |  | 1,043,150 |

PW/05

| ITEM | DESCRIPTION | QTY. | UNIT | RATE | SHS. |
| --- | --- | --- | --- | --- | --- |
| A B C | SUMMARY OF ELEMENTS PERIMETER WALL (PHASE 1) Substructures from page PW/03 Frame/Walling from page PW/04 Gates from page PW/05 |  |  |  | 1,443,060 1,490,800 1,043,150 |
|  | TOTAL FOR PERIMETER WALL CARRIED TO GRAND SUMMARY |  |  |  | 3,997,010 |

PW/06

BILL NO. 02:  BILLS OF QUANTITIES FOR THE PROPOSED SECURITY HOUSE

| ITEM | DESCRIPTION | QTY. | UNIT | RATE | SHS. |
| --- | --- | --- | --- | --- | --- |
| A B C D E F G H I J K L | GUARD HOUSE(PITCHED ROOF) SUBSTRUCTURES Allow for upholding the sides of excavations. Allow for keeping the excavations out of all kind of water by pumping or otherwise. Excavate to reduce level. Excavate top vegetable soil 200mm deep and deposit onsite not exceeding 100m away. Excavate normal soil from stripped level for trench not exceeding 1.5m deep. Extra over excavation for excavating in rock. Return fill and ram selected excavated material. Cart away extra excavated material as will be directed on site. Vibrated reinforced concrete 1:2:4 to foundations. 50mm (1:4:8) concrete blinding under foundations. 200mm thick masonry walling in cement/sand mortar (1:3) Approved insecticide treatment applied with manufacturer’s instructions ‘Gradiator or other equally approved. | - - 4 8 8 1 5 6 1 6 14 12 | ITEM ITEM CM SM CM CM CM CM CM SM SM SM | 1000 1000 150 150 300 1500 300 300 14000 600 1500 350 | 1,000 1,000 600 1,200 2,400 1,500 1,500 1,800 14,000 3,600 6,000 4,200 |
|  | - TOTAL CARRIED TO COLLECTION |  |  |  | 38,800 |

GH/1

| ITEM | DESCRIPTION | QTY. | UNIT | RATE | SHS. |
| --- | --- | --- | --- | --- | --- |
| A B C D E F G H I J K | 12mm diameter square twisted bars. 8mm ditto 300mm thick approved hardcore well watered and compacted in 100mm layers. 50mm thick murram blinding on top of compacted hardcore. 500 gauge damp proof membrane including end laps Damp proof course 200mm wide including end laps. 100mm thick vibrated reinforced concrete in slab. BRC reinforcement mesh ref.No.A98 sawn formwork to edge of floor slab 75 -150 high. 12mm thick cement/sand (1:4) rendering to building. 50mm in-situ concrete paving all around the building. Prepare and apply 3 coats of black bituminous paint to plinth. | 39 15 4 12 12 12 9 12 12 13 6 | KGS KGS CM SM SM LM SM SM SM SM SM | 200 200 1500 150 150 150 12000 350 400 800 350 | 7,800 3,000 6,000 1,800 1,800 1,800 108,000 4,200 4,800 10,400 2,100 |
|  | Total carried to collection COLLECTION Total from GH/1 Total brought down from above |  |  |  | 151,700 |
|  |  |  |  |  | 38,800 151,700 |
|  | - TOTAL FOR (SUBSTRUCTURES) CARRIEDTO SUMMARY |  |  |  | 190,500 |

GH/2

| ITEM | DESCRIPTION | QTY. | UNIT | RATE | SHS. |
| --- | --- | --- | --- | --- | --- |
| A B C D E F G H I J K | WALLING AND FRAME WORK 150mm thick walling in cement/sand 1:4 mortar Reinforced concrete 1:2:4 in ring beam 12mm diameter square twisted high tensile bars. 8mm ditto Sawn formwork to ring beam Extra over ditto for fair face Precast in permanent vents 300x225mm high with mosquito gauze Extra over walling for smooth stone dressing. Ditto for smooth vertical joints and recessed horizontal joints. 12mm thick cement and sand render with wood floor finish. Construct concrete work top 500x 2500mm with reinforced concrete all reinforcement’s bars and form work inclusive. | 36 1 56 27 13 12 4 48 48 7 2 | SM CM KGS KGS SM SM SM SM SM SM SM | 1200 14000 200 200 900 450 300 350 300 400 10000 | 43,200 14,000 11,200 5,400 11,700 5,400 1,200 16,800 14,400 2,800 20,000 |
|  | TOTAL FOR WALLING CARRIED TO SUMMARY |  |  |  | 146,100 |

GH/3

| ITEM | DESCRIPTION | QTY | UNIT | RATE | SHS. |
| --- | --- | --- | --- | --- | --- |
| A B C D E F G | ROOFING Construction All timber to be sawn cypress of G.S grade seasoned to equilibrium moisture content between 9% and 15% and to a requirement of K.S 02771 of 1991 treated with approved wood preservative. 100x50mm wall plate on and including 10mm cement and sand (1:4) mortar bed secured to reinforced concrete ring beam (m.s) by mild steel anchor bolts(m.s) The following in 56No. Nailed triangular timber trusses spanning clear 12700mm height 2500mm including hoisting and placing 3500mm above floor slab level 100x50mm trussed rafter 100x50mm tie beam 100x50mm strut/tie 75X50mm sawn celcured cypress purlins Ditto hip rafters Covering 30G roofing sheets Gauge 30 coloured corrugated iron sheets (MRM works) | 18 16 9 22 43 16 28 | LM LM LM LM LM LM SM | 350 350 350 350 250 250 1050 | 6,300 5,600 3,150 7,700 10,750 4,000 29,400 |
|  | TOTAL CARRIED TO COLLECTION |  |  |  | 66,900 |

GH/4

| ITEM | DESCRIPTION | QTY. | UNIT | RATE | SHS. |
| --- | --- | --- | --- | --- | --- |
| A B C D E | Ditto hip covers 450mm girth. Ditto ridge covers 300mm girth. Wrot cypress 25x200mm fascia board fixed to ends of rafters (m.s). Sundries 12mm diameter x 300mm long black mild steel anchor bolt embedded 180mm deep in ring beam at 1200mm including drilling holes in timber. 100x100x6mm thick galvanized mild steel nailing cleat once bent to form angle ten times drilled, one flange nailed to foot of rafter(m.s) other nailed to top of wall plate(m.s) | 8 5 22 14 10 | LM LM LM NO NO | 450 300 500 250 250 | 3,600 1,500 11,000 3,500 2,500 |
|  | CARRIED TO COLLECTION COLLECTION Total from GH/4 ………………………. Total from above ……………………… |  |  |  | 22,100 |
|  |  |  |  |  | 66,900 22,100 |
|  | TOTAL FOR ROOFING CARRIED TO SUMMARY |  |  |  | 89,000 |

GH/5

| ITEM | DESCRIPTION | QTY | UNIT | RATE | SHS |
| --- | --- | --- | --- | --- | --- |
| A B C D E F G | ELEMENT NO. 4 DOORS AND WINDOWS DOORS 1000x2100mm single leaf, outward-opening steel fabricated Paneled door complete with all hanging and fastening iron monger. WINDOWS Supply and fix Grilles window size 1500mm x 1200mm made of 20mmx20mmx2.0mm thick (1.13kg/m) Square Hollow Section tubes welded at 300mm centres both ways including ironmongeries. Ditto but weld mesh wire welded on to grilles windows framework. PAINTING Prepare and apply three coats high gloss paint on steel surfaces internally & externally Knot, prime and stop and apply three coats high gloss on fascia board. Ditto not exceeding 100-200mm girth. Doors both sides | 1 2 4 4 30 9 4 | NO NO SM SM LM LM SM | 10000 5000 600 400 150 100 400 | 10,000 10,000 2,400 1,600 4,500 900 1,600 |
|  | TOTAL OF ELEMENT NO.4 DOORS AND WINDOWS CARRIED TO SUMMARY |  |  |  | 31,000 |

GH/6

| ITEM | DESCRIPTION | QTY. | UNIT | RATE | SHS. |
| --- | --- | --- | --- | --- | --- |
| A B C D | FINISHES 12mm thick cement and sand plaster to walling. Prepare and apply 3 coats of plastic emulsion paint to walling. 40mm thick coloured screed finish steel trowelled smooth in red oxide. Prepare and apply 3 coats of oil paint to windows including burglar proofing | 72 37 11 6 | SM SM SM SM | 400 350 450 400 | 28,800 12,950 4,950 2,400 |
|  | TOTAL CARRIED TO SUMMARY SUMMARY Total from GH/2 (Substructures) Total from GH/3 (Walling/frame) Total from GH/5 (Roofing) Total from GH/6 (Doors/windows) Total from GH/7 above (Finishes) |  |  |  | 49,100 |
|  |  |  |  |  | 190,500 146,100 89,000 31,000 49,100 |
|  | TOTAL FOR GUARD HOUSE CARRIED TO GRAND SUMMARY |  |  |  | 505,700 |

GH/7

BILL NO. 03: PRIME COSTS AND PROVISIONAL SUMS

| ITEM | DESCRIPTION | SHS |
| --- | --- | --- |
| A B | PRIME COST AND PROVISIONAL SUMS PROVISIONAL SUMS Allow a Provisional Sum of Kenya Shillings One Hundred Fifty Thousand (Kshs.150,000.00) for contingencies. Allow A Provisional Sum of Kenya Shillings Fifty Thousand (Kshs.50,000.00) only for Project Management and Documentation. | 150,000 50,000 |
|  | TOTAL FOR PC & PROVISIONAL SUMS CARRIED TO GRAND SUMMARY | 200,000 |

PC&PS/01

BILLS OF QUANTITIES FOR THE PROPOSED CONSTRUCTION OF PERIMETER WALL & SECURITY HOUSE AT DEPUTY COUNTY COMMISSIONER’S RESIDENCE

GRAND SUMMARY

| ITEM | DESCRIPTION | For Official use only Kshs. | Cts. | For Contractor use only Kshs. | Cts. |
| --- | --- | --- | --- | --- | --- |
| A B. C | GRAND SUMMARY Perimeter wall Gate House PC and provisional sums | 3,997,010 505,700 200,000 | 00 00 00 |  |  |
|  | TOTAL ESTIMATED COST | 4,682,710 Say: 4,683,000 | 00 00 |  |  |

AMOUNT IN WORDS

Kenya Shillings Four Million, Six Hundred and Eighty Three Thousand

(Kshs. 4,683,000.00) only.

GS/01